Inhalt

1	Allgemein	1

1.1	Zweck	1

1.2	Ziel	2

1.3	Mitgeltende Dokumente	2

1.4	Begriffe	2

2	Anforderungen an den Anlieferzustand	2

2.1	Elektropolierte Rohre und Rohrleitungen	2

2.2	Sauberkeitsanforderungen (Life Science)	2

2.3	Oberflächenanforderungen	2

2.4	Ventile, Armaturen und medienführende Komponenten	3

2.5	Skids, Baugruppen und Module aus Edelstahl (gebürstete Oberflächen)	3

3	Verpackung	3

3.1	Schutz vor Witterung und Umwelteinflüssen	3

3.2	Korrosionsschutz	3

3.3	Transportbedingte Schäden	3

4	Prüfung und Abweichungen	4

4.1	Abweichungen	4

5	Verantwortung	4

6	Verbindlicher E-Mail-Betreff (Eingangsschlüssel)	4

6.1	Bedeutung der Felder im Betreff	4

6.2	Bauteil- und Komponentencodes (Auszug)	5

7	Gültigkeit	5

# Allgemein

Diese SMA beschreibt die verbindlichen Anforderungen an den Anlieferzustand, die Sauberkeit, den Oberflächenzustand, die Dokumentation sowie die BULK-/NON-BULK-Klassifizierung von Bauteilen im Anlagenbau.

Sie ist Bestandteil des integrierten Managementsystems (IMS) der SMB.

## Zweck

Zweck dieser Anweisung ist es,

einheitliche und verbindliche Anforderungen an Lieferanten festzulegen,

Qualitätsmängel bereits vor oder bei Anlieferung systematisch zu vermeiden,

die Einhaltung projektspezifischer, pharma- und life-science-relevanter Anforderungen sicherzustellen,

sowie eine klare Grundlage für Wareneingangsprüfung, Reklamationsmanagement und Lieferantensteuerung zu schaffen.

## Ziel

Ziel dieser Anweisung ist es, sicherzustellen, dass:

- alle gelieferten Bauteile frei von Korrosion, Beschädigungen und Kontaminationen sind,

- Oberflächen- und Sauberkeitsanforderungen eingehalten werden,

- Verpackung und Transport keinen negativen Einfluss auf Qualität oder Funktion haben,

- die Qualitätsdokumentation vollständig, korrekt und maschinenlesbar übermittelt wird,

- sowie eine eindeutige Zuordnung von Material, Bestellung, Position und Dokumentation gewährleistet ist.

## Mitgeltende Dokumente

- SAP-Standard-Bestelltext SMB

- Zeichnungen und technische Spezifikationen

- Projektspezifische Zusatzanforderungen

## Begriffe

- Life Science: Anwendungen mit erhöhten Anforderungen an Sauberkeit, Oberflächenqualität und Kontaminationsfreiheit (z. B. Pharma, Biotech).

- Optisch fehlerfrei: Frei von sichtbaren Beschädigungen, Korrosion oder Oberflächenfehlern.

- Rautiefe (Ra): Mittlere Oberflächenrauheit gemäß Bestellung, Zeichnung oder Spezifikation.

# Anforderungen an den Anlieferzustand

Alle Bauteile müssen anlagenbauspezifisch, sauber und optisch fehlerfrei angeliefert werden.

Unzulässig sind insbesondere:

- Dellen, Kratzer, Kerben, Schleif- oder Scheuerspuren,

- Korrosionsansätze oder Oxidationen,

- Verschmutzungen aus Fertigung, Bearbeitung oder Reinigung.

## Elektropolierte Rohre und Rohrleitungen

Für elektropolierte Rohre und Rohrleitungen gilt:

- Alle Rohrenden müssen dicht verschlossen sein (z. B. Kappen oder Stopfen).

- Der Verschluss muss das Eindringen von Schmutz, Feuchtigkeit und Fremdstoffen zuverlässig verhindern.

- Offene oder ungeschützte Rohrenden sind nicht zulässig.

## Sauberkeitsanforderungen (Life Science)

Für Life-Science- und qualitätskritische Anwendungen gilt:

Die Bauteile müssen frei sein von:

- Öl, Fett,

- Spänen, Metallabrieb,

- Staub, Partikeln,

- Reinigungs- oder Chemikalienrückständen,

- sonstigen Verunreinigungen.

Die Bauteile müssen für den Einsatz in Life-Science-, Pharma- oder Reinraumanwendungen geeignet sein.

## Oberflächenanforderungen

- Die geforderte Oberflächenqualität / Rautiefe (Ra-Werte) gemäß Bestellung, Zeichnung oder technischer Spezifikation ist verbindlich einzuhalten.

- Unzulässig sind Kratzer, Pickel, Poren, Korrosionsansätze oder Oberflächenfehler, die Reinigung, Qualifizierung oder Funktion beeinträchtigen.

## Ventile, Armaturen und medienführende Komponenten

Ventile, Armaturen und vergleichbare medienführende Komponenten müssen funktionsfähig sowie montagebereit geliefert werden. Bei vormontierten Einheiten gelten die Anforderungen für die gesamte

Baugruppe.

Zusätzlich sind folgende Punkte einzuhalten:

- Innen- und Außenflächen müssen sauber angeliefert werden.

- Alle medienberührten Öffnungen müssen dicht verschlossen sein.

- Alle geforderten Oberflächenqualitäten (z. B. Rautiefe) sind zwingend einzuhalten.

- Bulk-Material: Serienbasierte, austauschbare Komponenten.

- Non-Bulk-Material: Projektkritische, funktionsbezogene Komponenten mit erhöhter Dokumentationspflicht.

## Skids, Baugruppen und Module aus Edelstahl (gebürstete Oberflächen)

Für Skids, Baugruppen und Module mit gebürsteten Edelstahloberflächen (z. B. AISI 304 / 316L) gilt:

- Die Oberflächen müssen optisch einwandfrei angeliefert werden.

- Kratzer, Schleifspuren, Druck- oder Scheuerstellen sind nicht zulässig.

- Der definierte Bürstgrad / Schliff muss gleichmäßig und unverändert erhalten bleiben.

- Oberflächen sind so zu schützen, dass während Transport, Umschlag und Lagerung keine Beschädigungen entstehen.

- Schutzfolien dürfen bei Anlieferung weder beschädigt noch entfernt sein.

- Die Bewertung der Oberflächen erfolgt bei Tageslicht oder diffusem Licht.

- Auch rein optische Abweichungen gelten im Pharma- / Life-Science-Bereich als Qualitätsmangel.

# Verpackung

Verpackung und Transport sind so auszuführen, dass Korrosion, Kontamination und mechanische Beschädigungen unter allen üblichen Transportbedingungen ausgeschlossen sind, insbesondere bei winterlichen Straßenverhältnissen (Salznebel).

## Schutz vor Witterung und Umwelteinflüssen

- Bauteile, Baugruppen und Skids müssen vollständig gegen Witterungseinflüsse geschützt transportiert werden.

- Ein Transport ohne ausreichenden Wetterschutz (z. B. offener LKW ohne Plane oder geschlossenen Aufbau) ist nicht zulässig, sofern keine gleichwertige Schutzmaßnahme nachweislich sichergestellt ist.

- Verpackungen müssen so ausgeführt sein, dass sich kein Wasser oder salzhaltige Flüssigkeit unter Schutzfolien sammeln kann.

- Schutzfolien dürfen durch Fahrtwind oder Transportbewegungen nicht beschädigt oder gelöst werden.

## Korrosionsschutz

- Edelstahlbauteile müssen so geschützt sein, dass Kontakt mit salzhaltiger Feuchtigkeit ausgeschlossen ist.

- Erste Korrosionsansätze bei Anlieferung gelten als Qualitätsmangel.

## Transportbedingte Schäden

- Verpackung und Ladungssicherung müssen so ausgeführt sein, dass:

  - keine Reibung,

  - kein Scheuern,

  - keine punktuelle Belastung,

  - keine Verformung
auftreten kann.

- Die Wahl des Transportmittels ist dem Lieferanten überlassen, sofern die Einhaltung dieser Anforderungen sichergestellt ist.

Der Lieferant trägt die volle Verantwortung für:

- die fachgerechte Verpackung,

- den geeigneten Transport,

- den Schutz gegen Korrosion und Umwelteinflüsse,

- sowie die Einhaltung dieser Liefervorschrift

bis zum Gefahrenübergang gemäß dem in der Bestellung vereinbarten Incoterm (z. B. DAP, FCA, EXW o. ä.).

Unabhängig vom Gefahrenübergang bleiben Mängel aufgrund mangelhafter Verpackung, unzureichendem Korrosionsschutz oder fehlerhafter Verarbeitung reklamationsfähig.

# Prüfung und Abweichungen

SMB ist berechtigt, bei Anlieferung Sicht-, Sauberkeits- und Oberflächenprüfungen durchzuführen sowie Verschlüsse, Schutzmaßnahmen und Oberflächenzustände zu bewerten.

## Abweichungen

Abweichungen von dieser Liefervorschrift führen zu einer Liefermangelmeldung (Reklamation).

Bei Life-Science-relevanten Bauteilen, Ventilen oder Skids kann dies zusätzlich zur Zurückweisung der Lieferung oder zur Sperrung bis zur Klärung führen.

# Verantwortung

| Lieferant |  |  |  |  |  | Einhaltung dieser Liefervorschrift |
| --- | --- | --- | --- | --- | --- | --- |
| Einkauf SMB |  |  |  |  |  | Verankerung im SAP-Bestelltext, Reklamationen |
| QM / QS SMB |  |  |  |  |  | Prüfung, Bewertung, |
| Projekt / Baustelle |  |  |  |  |  | Rückmeldung projektspezifischer Abweichungen |

# Verbindlicher E-Mail-Betreff für die Enddokumentation

Der E-Mail-Betreff ist verpflichtend als eindeutiger, maschinenlesbarer Eingangsschlüssel zu verwenden.

Verbindliches Betreff-Schema:
MATNR_BESTNR_POSNR_CODE_DOCTYPE_CHARGENNR

Beispiel:
4711_4500123456_0010_BRO_31_H12345

Abweichungen vom Betreff-Schema führen zur Nichtzuordnung der Dokumentation und gelten als nicht ordnungsgemäß übermittelt.

## Bedeutung der Felder im Betreff

| Feld | Bedeutung |
| --- | --- |
| MATNR | Materialnummer gemäß Bestellung |
| BESTNR | SAP-Bestellnummer |
| POSNR | Positionsnummer der Bestellung |
| CODE | Bauteil-/Komponentencode gemäß Abschnitt 8.4 |
| DOCTYPE | Dokumentenart (z. B. 31 = Werkszeugnis 3.1) |
| CHARGENNR | Heat, Chargen- oder Losnummer |

## Bauteil- und Komponentencodes (Auszug)

Die folgenden Codes sind für den E-Mail-Betreff verbindlich zu verwenden:

A. BULK-CODES

| Code | Bauteil |
| --- | --- |
| BRO | Rohr |
| BBO | Rohrbogen |
| BRE | Reduktion |
| BTE | T-Stück |
| BKA | Kappe |
| BFL | Flansch |
| BDI | Dichtung |
| BSC | Schrauben / Verbindungsmaterial |
| BST | Stutzen |
| BHA | Halterung |
| BSP | Stahlprofil |
| BIS | Isolierung |
| BRV | Rohrverschraubungen |

B. NON-BULK-CODES

| Code | Bauteil |
| --- | --- |
| NBV | Ventil / Armatur |
| NBI | Instrument |
| NBF | Filter |
| NBW | Wärmetauscher |
| NBB | Behälter |
| NBS | Skid |
| NBG | Baugruppe |
| NBD | Zeichnungsteil |
| NBM | Modul |

C. Dokumententypen (DOCTYPE)

| Code | Bedeutung |
| --- | --- |
| 31 | Werkszeugnis 3.1 |
| 32 | Werkszeugnis 3.2 |
| CoC | Konformitätsbescheinigung |
| DRW | Zeichnung |
| WPS | Schweißanweisung |
| WPQR | Schweißverfahrensprüfung |
| DOK | Sonstige Dokumentation |

# Gültigkeit

Diese Liefervorschrift tritt mit Veröffentlichung in Kraft und gilt für alle laufenden und zukünftigen Bestellungen, sofern im Bestelltext darauf verwiesen wird.

Contents

1	General	1

1.1	Purpose	1

1.2	Objective	2

1.3	Related documents	2

1.4	Terms	2

2	Requirements for the delivery condition	2

2.1	Electropolished pipes and piping	2

2.2	Cleanliness requirements (life sciences)	2

2.3	Surface requirements	2

2.4	Valves, fittings, and media-carrying components	3

2.5	Skids, assemblies, and modules made of stainless steel (brushed surfaces)	3

3	Packaging	3

3.1	Protection against weather and environmental influences	3

3.2	Corrosion protection	3

3.3	Transport-related damage	3

4	Inspection and deviations	4

4.1	Deviations	4

5	Responsibility	4

6	Mandatory email subject line (incoming key)	4

6.1	Meaning of the fields in the subject line	4

6.2	Part and component codes (excerpt)	5

7	Validity	5

# General

This SMA describes the mandatory requirements for the delivery condition, cleanliness, surface condition, documentation, and BULK/NON-BULK classification of components in plant engineering.

It is part of SMB's integrated management system (IMS).

## Purpose

The purpose of this instruction is to

to establish uniform and binding requirements for suppliers,

Systematically avoiding quality defects before or upon delivery,

ensuring compliance with project-specific, pharmaceutical, and life science-related requirements,

and create a clear basis for incoming goods inspection, complaint management, and supplier control.

## Objective

The objective of this instruction is to ensure that:

- all delivered components are free of corrosion, damage, and contamination,

- surface and cleanliness requirements are met,

- packaging and transport have no negative impact on quality or function,

- quality documentation is transmitted in a complete, correct, and machine-readable form,

- and that there is a clear assignment of material, order, position, and documentation.

## Applicable documents

- SAP standard order text SMB

- Drawings and technical specifications

- Project-specific additional requirements

## Terms

- Life science: Applications with increased requirements for cleanliness, surface quality, and freedom from contamination (e.g., pharmaceuticals, biotech).

- Visually flawless: Free from visible damage, corrosion, or surface defects.

- Roughness (Ra): Average surface roughness according to order, drawing, or specification.

# Requirements for the delivery condition

All components must be delivered in a condition suitable for plant construction, clean and visually flawless.

The following are not permitted in particular:

- Dents, scratches, nicks, grinding or scuff marks,

- corrosion or oxidation,

- Contamination from manufacturing, processing, or cleaning.

## Electropolished pipes and tubing

The following applies to electropolished pipes and tubes:

- All pipe ends must be sealed tightly (e.g., with caps or plugs).

- The closure must reliably prevent the ingress of dirt, moisture, and foreign substances.

- Open or unprotected pipe ends are not permitted.

## Cleanliness requirements (life sciences)

The following applies to life science and quality-critical applications:

The components must be free of:

- Oil, grease,

- chips, metal abrasion,

- dust, particles,

- cleaning agent or chemical residues,

- other contaminants.

The components must be suitable for use in life science, pharmaceutical, or clean room applications.

## Surface requirements

- The required surface quality/roughness (Ra values) as specified in the order, drawing, or technical specification must be strictly adhered to.

- Scratches, pits, pores, corrosion spots, or surface defects that impair cleaning, qualification, or function are not permitted.

## Valves, fittings, and media-carrying components

Valves, fittings, and comparable media-carrying components must be delivered in a functional and ready-to-install condition. For pre-assembled units, the requirements apply to the entire

assembly.

In addition, the following points must be observed:

- Internal and external surfaces must be delivered clean.

- All openings that come into contact with media must be tightly sealed.

- All required surface qualities (e.g., roughness) must be strictly adhered to.

- Bulk material: Series-based, interchangeable components.

- Non-bulk material: Project-critical, function-related components with increased documentation requirements.

## Skids, assemblies, and modules made of stainless steel (brushed surfaces)

The following applies to skids, assemblies, and modules with brushed stainless steel surfaces (e.g., AISI 304/316L):

- The surfaces must be delivered in perfect visual condition.

- Scratches, grinding marks, pressure marks, or scuffs are not permitted.

- The defined degree of brushing/grinding must remain uniform and unchanged.

- Surfaces must be protected so that no damage occurs during transport, handling, and storage.

- Protective films must not be damaged or removed upon delivery.

- Surfaces are evaluated in daylight or diffuse light.

- Even purely visual deviations are considered quality defects in the pharmaceutical/life sciences sector.

# Packaging

Packaging and transport must be carried out in such a way that corrosion, contamination, and mechanical damage are ruled out under all normal transport conditions, especially in winter road conditions (salt spray).

## Protection against weather and environmental influences

- Components, assemblies, and skids must be transported fully protected against weather influences.

- Transport without adequate weather protection (e.g., open truck without tarpaulin or closed body) is not permitted unless equivalent protective measures can be demonstrably ensured.

- Packaging must be designed in such a way that no water or salty liquids can collect under protective films.

- Protective films must not be damaged or loosened by airflow or transport movements.

## Corrosion protection

- Stainless steel components must be protected in such a way that contact with salty moisture is impossible.

- Initial signs of corrosion upon delivery are considered a quality defect.

## Transport-related damage

- Packaging and load securing must be carried out in such a way that:

  - no friction,

  - no chafing,

  - no point loads,

  - no deformation
can occur.

- The choice of means of transport is left to the supplier, provided that compliance with these requirements is ensured.

The supplier bears full responsibility for:

- proper packaging,

- suitable transport,

- protection against corrosion and environmental influences,

- and compliance with these delivery instructions

until the transfer of risk in accordance with the Incoterm agreed in the order (e.g., DAP, FCA, EXW, or similar).

Regardless of the transfer of risk, defects due to inadequate packaging, insufficient corrosion protection, or faulty workmanship remain subject to complaint.

# Inspection and deviations

SMB is entitled to carry out visual, cleanliness, and surface inspections upon delivery and to evaluate seals, protective measures, and surface conditions.

## Deviations

Deviations from these delivery regulations will result in a notification of delivery defect (complaint).

In the case of life science-related components, valves, or skids, this may also result in the rejection of the delivery or in it being blocked until clarification has been provided.

# Responsibility

| Supplier |  |  |  |  |  | Compliance with these delivery regulations |
| --- | --- | --- | --- | --- | --- | --- |
| Purchasing SMB |  |  |  |  |  | Anchoring in SAP order text, complaints |
| QM / QS SMB |  |  |  |  |  | Inspection, evaluation, |
| Project / Construction site |  |  |  |  |  | Feedback on project-specific deviations |

# Mandatory email subject line (incoming key)

The email subject line must be used as a unique, machine-readable entry key.

Mandatory subject line format:
MATNR_BESTNR_POSNR_CODE_DOCTYPE_CHARGENNR

Example:
4711_4500123456_0010_BRO_31_H12345

Deviations from the subject line format will result in the documentation not being assigned and will be considered improperly transmitted.

## Meaning of the fields in the subject line

| Field | Meaning |
| --- | --- |
| MATNR | Material number according to order |
| BESTNR | SAP order number |
| POSNR | Item number of the order |
| CODE | Part/component code according to section 8.4 |
| DOCTYPE | Document type (e.g., 31 = factory certificate 3.1) |
| CHARGENNR | Heat, batch, or lot number |

## Part and component codes (excerpt)

The following codes must be used in the email subject line:

A. BULK CODES

| Code | Part |
| --- | --- |
| BRO | Pipe |
| BBO | Pipe bend |
| BRE | Reducer |
| BTE | T-piece |
| BKA | Cap |
| BFL | Flange |
| BDI | Gasket |
| BSC | Screws / Fasteners |
| BST | Nozzle |
| BHA | Bracket |
| BSP | Steel profile |
| BIS | Insulation |
| BRV | Pipe fittings |

B. NON-BULK CODES

| Code | Component |
| --- | --- |
| NBV | Valve / Fitting |
| NBI | Instrument |
| NBF | Filter |
| NBW | Heat exchanger |
| NBB | Tank |
| NBS | Skid |
| NBG | Assembly |
| NBD | Drawing part |
| NBM | Module |

C. Document types (DOCTYPE)

| Code | Meaning |
| --- | --- |
| 31 | Factory certificate 3.1 |
| 32 | Factory certificate 3.2 |
| CoC | Certificate of conformity |
| DRW | Drawing |
| WPS | Welding procedure specification |
| WPQR | Welding procedure qualification |
| DOK | Other documentation |

# Validity

These delivery terms shall come into effect upon publication and shall apply to all current and future orders, provided that reference is made to them in the order text.